# Ex7 Report | ESP32 multi-task with RTOS

Nguyen Tieu Phuong 20210692

Wokwi simulation link: https://wokwi.com/projects/414942789273379841

Source code:

#include <WiFi.h>

#include <HTTPClient.h>

#include <DHTesp.h>

#include "ArduinoJson.h"

// Pin Definitions

#define LED_PIN 32

#define DHT_PIN 15

#define PIR_SENSOR 12

// WiFi credentials

const char* ssid = "Wokwi-GUEST";

const char* password = "";

// Server URL

String serverPostUrl = "https://postman-echo.com/post";

// Objects

DHTesp dhtSensor;

// Variables

float temperature = 0.0;

float humidity = 0.0;

int motion = 0;

// // Task Handles

// TaskHandle_t blinkTaskHandle;

// TaskHandle_t sensorTaskHandle;

// TaskHandle_t sendTaskHandle;

// Function Prototypes

void blinkTask(void* parameter);

void sensorTask(void* parameter);

void sendTask(void* parameter);

void setup() {

Serial.begin(9600);

// WiFi connection

Serial.print("Connecting to WiFi");

WiFi.begin(ssid, password);

while (WiFi.status() != WL_CONNECTED) {

delay(100);

Serial.print(".");

}

Serial.println("\nWiFi Connected!");

// Pin configurations

pinMode(LED_PIN, OUTPUT);

pinMode(PIR_SENSOR, INPUT);

// Initialize DHT sensor

dhtSensor.setup(DHT_PIN, DHTesp::DHT22);

// Create FreeRTOS tasks

xTaskCreate(blinkTask, "Blink LED", 1024, NULL, 1, NULL);

xTaskCreate(sensorTask, "Read Sensors", 2048, NULL, 1, NULL);

xTaskCreate(sendTask, "Send Data", 8192, NULL, 1, NULL);

}

void loop() {

// Empty, FreeRTOS handles the tasks

}

void blinkTask(void* parameter) {

for(;;){ // infinite loop

// Turn the LED on

digitalWrite(LED_PIN, HIGH);

// Pause the task for 500ms

vTaskDelay(500 / portTICK_PERIOD_MS);

// Turn the LED off

digitalWrite(LED_PIN, LOW);

// Pause the task again for 500ms

vTaskDelay(500 / portTICK_PERIOD_MS);

// Serial.println("LED is blinking ...");

}

}

void sensorTask(void* parameter) {

for(;;) {

TempAndHumidity data = dhtSensor.getTempAndHumidity();

temperature = data.temperature;

humidity = data.humidity;

// Read motion sensor

motion = digitalRead(PIR_SENSOR);

if (motion == HIGH) {

Serial.println("Motion detected");

} else {

Serial.println("No motion");

}

// Log sensor values

Serial.println("Temperature: " + String(temperature) + "°C");

Serial.println("Humidity: " + String(humidity) + "%");

Serial.println("---");

vTaskDelay(2000 / portTICK_PERIOD_MS); // Update every 2 seconds

}

}

void sendTask(void* parameter) {

for(;;) {

if (WiFi.status() == WL_CONNECTED) {

HTTPClient http;

http.begin(serverPostUrl);

http.addHeader("Content-Type", "application/json");

// Create JSON object

DynamicJsonDocument doc(1024);

String jsonStr;

doc["temperature"] = temperature;

doc["humidity"] = humidity;

doc["motion"] = motion;

serializeJson(doc, jsonStr);

// Send POST request

int httpResponseCode = http.POST(jsonStr);

if (httpResponseCode > 0) {

Serial.print("HTTP Response code: ");

Serial.println(httpResponseCode);

String payload = http.getString();

Serial.println("Response:");

Serial.println(payload);

} else {

Serial.print("Error code: ");

Serial.println(httpResponseCode);

}

http.end();

} else {

Serial.println("WiFi Disconnected");

}

vTaskDelay(5000 / portTICK_PERIOD_MS); // Send data every 5 seconds

}

}

Screenshots of running program: