Ex 7. ESP32 multi-tasks with FreeRTOS

Write programs for ESP32 using multi-tasking with FreeRTOS. The program has 3 tasks:

Task 1: blink LED

Task 2: Read data from sensors (DHT, PIR motion sensors as in exercise 4)

Task 3: Send the sensors data to a server (postman server as in exercise 5)

Using wokwi https://wokwi.com/

Using the server api:

https://postman-echo.com/get

https://postman-echo.com/post

Sample code:

#include "DHT.h"

#include <HTTPClient.h>

#include <WiFi.h>

#include <string.h>

const char* ssid = "HUNG-DELLPC";

const char* password = "hung1234" ;

#define DHT_PIN 15

#define DHT_TYPE DHT22

//Task 1: blink LED

void BlinkLED( void *pvParameters );

//Task 2: Read data from sensors

void ReadSensors( void *pvParameters );

//Task 3: Send data to server

void SendData(void *pvParameters);

const int led1 = 2; // Pin of the LED

DHT dht = DHT(DHT_PIN, DHT_TYPE);

void setup(){

pinMode(led1, OUTPUT);

dht.begin();

Serial.begin(9600);

Serial.println("Multi-tasking with FreeRTOS ...");

//setup wifi

WiFi.begin(ssid,password);

Serial.println("conecting");

while(WiFi.status() != WL_CONNECTED)

{

delay(500);

Serial.print(".");

}

Serial.println("");

Serial.print("Connected to WiFi network with IP Address: ");

Serial.println(WiFi.localIP());

//Create the task 1 to run BlinkLED

xTaskCreate(

BlinkLED,    // Function that should be called

"Blink LED",   // Name of the task (for debugging)

1024,            // Stack size (bytes)

NULL,            // Parameter to pass

1,               // Task priority

NULL             // Task handle

);

//TODO: create task 2, task 3

}

//The function for Task 1

void BlinkLED( void *pvParameters ){

for(;;){ // infinite loop

// Turn the LED on

digitalWrite(led1, HIGH);

// Pause the task for 500ms

vTaskDelay(500 / portTICK_PERIOD_MS);

// Turn the LED off

digitalWrite(led1, LOW);

// Pause the task again for 500ms

vTaskDelay(500 / portTICK_PERIOD_MS);

Serial.println("LED is blinking ...");

}

}

//The function for Task 2

void ReadSensors( void *pvParameters ){

//TODO

}

void SendingData(void* pvParameters){

//TODO

}

void loop() {

// put your main code here, to run repeatedly:

}

Submission:

Write a report with:

- The code of programs with suitable comments

- Screen photos when running programs

- Link of simulations with wokwi